Marshal Simons

Dr. Tyler Flaagan

INFA-735

18 February 2025

Lab 04 – IoT Exploitation Research

A joke passed around the security community about Internet of Things (IoT) devices is that the ‘s’ in IoT stands for security. Well, one may think that doesn’t make sense because there is no ‘s’ in IoT, which is the point; security is often lacking or nonexistent in many of these devices. IoT exploitation has quickly become a hot topic in the last decade, and due to their inexpensive cost, it is easy to pick up a cheap Wi-Fi camera from Amazon or Temu to poke at.

And poking at them is precisely what many people have done, which subsequently led to numerous critical vulnerabilities being found that can be easily exploited. The methods people use to find and exploit these vulnerabilities vary with the target and the operator. However, two common ways attackers exploit IoT devices are reverse engineering the firmware and weak configurations, often through default credentials.

Firmware reverse engineering

Firmware reverse engineering is a method of exploitation by utilizing an IoT device’s firmware to find the code detailing the operation of the device and the potential to discover vulnerabilities, modify its functionality, or even extract sensitive information such as credentials.  Getting hands on a device’s firmware can be done in various ways. The easiest way is to download the firmware driver from the internet. This may not always be possible but cheaper devices will commonly lack the ability for vendors to push updates directly to the device. This leads to the responsibility being handed to the user, who is unlikely to do so.

The more hands-on method requires a few specialized but, thankfully, relatively cheap and accessible tools. While this paper will focus more on already having an unencrypted firmware image, it is important to briefly summarize the process of extracting firmware directly from the device as it may be the only option in some situations.

At DEF CON 32 in 2024, Andrew Bellini (DigitalAndrew), an Electrical Engineer at TCM Security, gave a presentation that steps through extracting firmware via UART debugging ports. UART, or Universal Asynchronous Receiver/Transmitter, is an old and ubiquitous protocol used for communication via changes in voltages to represent binary digits. Using a UART to USB bridge that can be purchased for a few dollars, a connection can be made to the IoT device using a terminal emulator such as screen on Linux.

Next, the firmware can finally be extracted and sent to the operator’s host machine. The Trivial File Transfer Protocol (TFPT) can transfer the entire root file system back onto the device if a shell is acquired. In the case of file transfer not being an option, the possibility of reading the firmware directly from the ROM still exists. This is done by connecting a flash reader to the ROM chip on the board back to the operator's host machine, where they can then use a tool such as flashrom, which will read out the firmware.

With the firmware in hand, the time has come to reverse engineer it to see what useful information can be found. Kavishka Gihan, a cybersecurity researcher, published an article on Medium in 2021 detailing the steps to reverse engineering a piece of firmware successfully. To demonstrate the application of IoT reverse engineering, the examples below follow their guide utilizing the same TP-Link TL-WR1042NDv1 router firmware.

Firstly, to understand what to do with the file, it is essential to know its type. The file tool will check the file headers, if present, to determine its type.

As expected, we find the file is indeed firmware, along with information on the vendor and firmware version. Next, a tool named Binwalk can search for the file's contents.

Here, Binwalk found three important pieces of information: two LZMA compressed data locations (yellow) and the Squashfs filesystem (red) with their respective locations in memory displayed by byte offsets. However, for this example, only the Squashfs filesystem will be analyzed.

From here, the memory offsets can be used to extract the data located there. Executing the command provided by Gihan utilizes the dd data duplication tool to skip to the desired location in memory and then output one block size into the output file.

Now that the Squashfs filesystem has been pulled from the firmware, the unsquashfs tool can be utilized to see what it contains before extracting the data. Peaking inside displays the typical Linux file system expected with IoT devices.

Finally, a last command using the same toolset means the device's Linux system has successfully been extracted.

With the file system acquired, analysis can now be performed to see what information it contains. Looking into the directory, the shadow file can be found and read from where the hashed passwords for the root and Admin users are contained.

The file system also contains the binary files associated with the device in the /bin directory. While it is possible to reverse engineer each binary, a tool named Bugprove can automatically evaluate the binary files. Bugprove can also do all the previous steps by uploading the original firmware image, which will perform a vulnerability assessment. The report contains information regarding applicable CVEs and CWEs, Unsafe Function Calls, and extracting the entire file system where the shadow file can be found again.  The picture below displays the assessment for the same firmware image used in the previous examples.

Exploiting weak configurations in smart devices

One of the most common vulnerabilities attackers exploit with IoT devices is default/weak credentials. Often, devices will ship with default credentials that can be found online or by cracking found hash values, as in the previous example. These default credentials are also unlikely to be changed from their default credentials, leaving them vulnerable to exploitation.

Common protocols used for IoT communication include Telnet and SSH. If an attacker gains access to the username and password as a superuser, they will have a much easier time acquiring a shell on the target machine. For example, assuming the router used in the previous example used default credentials, an attacker could search for similar devices using a tool such as Shodan. Then, they could attempt to open a ssh or telnet connection using the default credentials.  Since users are unlikely to change their password from the default, their likelihood of success is likely high.

Given how many smart devices are within some households and the previously stated use of weak default credentials, an attacker could also exploit these to move around inside the home. In this scenario of initial access gained through the router, further network enumeration using a Nmap scan through an SSH tunnel may reveal more IoT devices, such as keyless entry locks that could open the front door or view the camera commonly found on them. If such a device was found, an attacker could follow a similar process of trying default credentials or getting their hands on the firmware to reverse engineer it.

References

DEFCONConference. (2024, October 16). DEF CON 32 - Anyone can hack IoT- Beginner’s Guide to Hacking Your First IoT Device - Andrew Bellini. YouTube. https://www.youtube.com/watch?v=YPcOwKtRuDQ

Gihan, K. (2023, May 13). IoT hacking — Reversing a router firmware. Medium. https://kavigihan.medium.com/iot-hacking-reversing-a-router-firmware-df6e06cc0dc9

Lim, B. (2023, August 16). Reverse Engineering IoT: Firmware Extraction. West Side Electronics; West Side Electronics. https://westsideelectronics.com/reverse-engineering-firmware/

Wilson, Marc. “Setup & Configure TFTP on Windows 10 [ Free Server Tool Download ].” Websentra, 19 Nov. 2020, www.websentra.com/tftp-on-windows-10/. Accessed 8 Apr. 2025.